U.S. Export Control — Order of Review

ITAR / EAR Classification Sequence Reference

Interactive OOR: exportcompliance.help → Training → Order of Review  (clickable flowchart with direct chapter links)

This document is a printed reference companion. Use the interactive site version during active classification work.

| Step | Question / Action | If YES | If NO |
| --- | --- | --- | --- |
| PART I — ITAR (22 CFR Parts 120–130) |  |  |  |
| 1 | Is the item enumerated (explicitly listed) in a USML category? 22 CFR Part 121 | ✓ Subject to ITAR. Classify under corresponding USML category. STOP. | → No: continue |
| 2 | Is the item described by a USML catch-all (specially designed) paragraph? 22 CFR 120.41 | ✓ Subject to ITAR. Classify under corresponding USML category. STOP. | → No: continue |
| PART II — EAR (15 CFR Parts 730–774) |  |  |  |
| 3 | Continue to the EAR — determine the appropriate CCL category (0–9). |  |  |
| 4 | Identify the CCL product group: A = Systems/Equipment/Components · B = Test/Inspection/Production · C = Materials · D = Software · E = Technology |  |  |
| 5 | Is the item enumerated in a 600 series or 9x515 ECCN? 600 series; 9x515 | ✓ Subject to EAR. Classify under corresponding 600 series / 9x515 ECCN. | → No: continue |
| 6 | Is the item described by a 600 series or 9x515 catch-all (specially designed) paragraph? 600 series .x; 9x515 | ✓ Subject to EAR. Classify under corresponding 600 series / 9x515 ECCN. | → No: continue |
| 7 | Is the item enumerated in any other ECCN? 15 CFR 774 | ✓ Subject to EAR. Classify under corresponding ECCN. | → No: continue |
| 8 | Is the item described by any other ECCN catch-all paragraph using 'specially designed'? 15 CFR 772.1 | ✓ Subject to EAR. Classify under corresponding ECCN. | → No: continue |
| 9 | EAR99 — the item is subject to the EAR but not described by any specific ECCN. Note: EAR99 is not restriction-free — OFAC screening, end-use / end-user controls (Part 744), and country-based restrictions (Part 746) still apply. |  |  |

Shortcut note: If it is obvious the item does not fit any CCL category or product group, you may conclude EAR99 directly (e.g., a box of milk). Due diligence through the full sequence is strongly recommended.

| AFTER CLASSIFICATION — What comes next |  |
| --- | --- |
| License determination | Use the ECCN's Reasons for Control + Commerce Country Chart (CCC) to check NLR eligibility for the destination. |
| End-use / end-user controls | Check Part 744 separately — a clean CCC result does not rule out end-use or end-user license requirements. |
| License exceptions | If NLR does not apply, check whether a Part 740 exception authorizes the transaction. Review 15 CFR 740.2 restrictions first. |
| License application | If neither NLR nor an exception applies, a BIS license (SNAP-R) or ITAR authorization (DECCS) is required. |
| OFAC screening | All transactions require denied party screening regardless of ECCN or authorization path. |
| Recordkeeping | Document the full OOR analysis. Retain records 5 years from last export of the item. Use the Classification Record template. |

Order of Review Reference  —  For informational purposes only. Consult qualified legal counsel for specific classification questions.